CPE301 – SPRING 2019

# MIDTERM 2

Student Name: Victor Martinez

Student #: 5003131674

Student Email: martiv6@unlv.nevada.edu

Primary Github address: https://github.com/martiv6/

Directory: https://github.com/martiv6/midterms/Midterm_2

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ESP-01

APDS

9960

BREAD

BOARD

328P

Mini

POWERSUPPLY

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* MIDTERM_2.c

*

* Created: 5/01/2019 10:06:39 PM

* Author : victor

*/

#define F_CPU 16000000UL

#define BAUD 9600

#define MYUBRR F_CPU/16/BAUD-1

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

#include <stdlib.h>

#include "I2C_MASTER_C.h"

#include "APDS_9960_LIB.h"

void USART_tx_string(char *data); 										// where it prints string USART

void USART_init( unsigned int ubrr ); 									// where USART initializes

void RGB_dect();														// used to read the colors and put their values

void APDS_9960_init();													// begins to use APDS sensor

uint16_t baud_rate = MYUBRR;		// Int declaration

uint8_t red_low, red_high;		//

uint8_t blue_low, blue_high;	//

uint8_t green_low, green_high;	//

uint8_t config;					//

char outs[256];															// room we have to print

int main(void)

{

uint16_t red = 0;														// Declare red

uint16_t green = 0;														// Declare green

uint16_t blue = 0;														// Declare blue

i2c_init();																// i2C function call

USART_init(MYUBRR);														// uart initialization function call

APDS_9960_init();														// initialize 9960 sensor function call

unsigned char AT_CHECK[] = "AT\r\n"; 									// AT Commands

unsigned char CWMODE_SET[] = "AT+CWMODE=1\r\n"; 						// Set MODE

unsigned char CWJAP_LOGIN[] = "AT+CWJAP=\"iPXSMax\",\"12345678\"\r\n"; 	// MUST CHANGE WIFI AND PASSWORD

unsigned char CIPMUX_SET[] = "AT+CIPMUX=0\r\n";

unsigned char CIPSTART_SEND[] = "AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n";

unsigned char CIPSEND_FINISH[] = "AT+CIPSEND=100\r\n";

_delay_ms(2500);

USART_tx_string(AT_CHECK); 													//send commands

_delay_ms(6000);

USART_tx_string(CWMODE_SET); 												//set mode

_delay_ms(6000);

USART_tx_string(CWJAP_LOGIN); 												//connect to Wifi

_delay_ms(16000);

while (1)

{

///////////////////////// SENDING RED, GREEN, BLUE READINGS/////////////////////////

USART_tx_string(CIPMUX_SET); 												//select MUX

_delay_ms(10000);

USART_tx_string(CIPSTART_SEND);												//connect TCP

_delay_ms(10000);

USART_tx_string(CIPSEND_FINISH);											//send size

_delay_ms(6000);

RGB_dect(&red, &blue, &green);												// Call colors function

snprintf(outs,sizeof(outs),"GET https://api.thingspeak.com/update?api_key=5HICIFGYJQ1XUU90&field1=0%05u&field2=%05u&field3=%05u\r\n", red,green,blue); // print it

USART_tx_string(outs);														//send data

_delay_ms(10000);

}

}

/* INIT USART (RS-232) */

void USART_init( unsigned int ubrr ) {

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B |= (1 << TXEN0) | (1 << RXEN0)| ( 1 << RXCIE0); 				// Enable receiver, transmitter & RX interrupt

UCSR0C |= (1<<UCSZ01) | (1 << UCSZ00);

}

void USART_tx_string( char *data ) 										// used to print on to the screen

{

while ((*data != '\0')) {

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

void RGB_dect(uint16_t *red, uint16_t *blue, uint16_t *green)	 // Function used to read the colors for sensor (red, blue and green)

{

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &red_low, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &red_high, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &green_low, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &green_high, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &blue_low, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &blue_high, 1);

*red = red_high << 8 | red_low;

*blue = blue_high << 8 | blue_low;

*green = green_high << 8 | green_low;

}

void APDS_9960_init()	// Function used to initialize the Sensor only for the RGB, no gesture or proximity used

{

i2c_readReg(APDS_WRITE, APDS9960_ID, &config,1);

if(config != APDS9960_ID_1)

while(1)

{

config = 1 << 1 | 1 << 0 | 1 << 3 | 1 << 4;

}

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &config, 1);

config = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &config, 1);

config = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &config, 1);

config = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &config, 1);

config = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &config, 1);

config = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &config, 1);

config = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &config, 1);

config = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &config, 1);

config = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &config, 1);

config = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &config, 1);

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Fwgln-hfr2Q

- GITHUB LINK OF THIS DA

https://github.com/martiv6/midterms/Midterm_2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Victor Martinez